# VL53L1X Multi-Sensor Setup Guide

# 1. Hardware Connection Mapping

Each sensor requires power (3.3V), Ground, and a dedicated pair of SDA/SCL pins. For this 4-sensor configuration, the default I2C bus is used plus three additional software-defined buses.

| Sensor | Component | Pi’s Physical Pin | RPi Function |
| --- | --- | --- | --- |
| All Sensors | VDD (Pin 5) | Pin 1 | 3.3V Power |
| All Sensors | GND (Pin 6) | Any GND Pin | Ground |
| Sensor 1 | SDA / SCL | Pin 3 / Pin 5 | I2C1 (Standard) |
| Sensor 2 | SDA / SCL | Pin 7 / Pin 29 | I2C3 (Modified) |
| Sensor 3 | SDA / SCL | Pin 32 / Pin 33 | I2C5 (Modified) |
| Sensor 4 | SDA / SCL | Pin 15 / Pin 16 | I2C6 (Modified) |

Note: On the sensor PCB, the square pin indicates PIN 1 in the datasheet.

# 2. Enabling Additional I2C Buses

To use four sensors, modify the system configuration to enable extra I2C interfaces:

1. Open terminal and run: sudo nano /boot/firmware/config.txt

2. Add these lines to the end of the file:

# Enable additional I2C buses (Avoiding SPI pins)
dtoverlay=i2c3,pins_4_5
dtoverlay=i2c5,pins_12_13
dtoverlay=i2c6,pins_22_23

3. Reboot the Raspberry Pi.

# 3. Verification

Check for address 0x29 on each bus using:

sudo i2cdetect -y 1

sudo i2cdetect -y 3

sudo i2cdetect -y 5

sudo i2cdetect -y 6

# 4. Software Execution

Clone the repository and run the multi-sensor test:

git clone https://github.com/slaghuis/VL53L1X-Raspberry-Pi.git
cd ~/your /directory /whatever 
g++ test_multi.cpp vl53l1x_multi.cpp -o test_multi
./test_multi